# ELEC 594 Spring 2026

SmartCam Automatic Emergency Braking (AEB) System Documentation

Department of Electrical and Computer Engineering

## Test & Documentation: Ashley Garcia & Mehul Goel

Phase 1. System Architecture Overview

- The system is divided into two layers to ensure the Raspberry Pi 5's new RP1 GPIO architecture is handled correctly without "Environment Poisoning" from old ROS distributions.

- Host Layer (Debian): Runs a lightweight Python script using the lgpio library to read the HC-SR04. It broadcasts distance data via a UDP Socket (low latency).

- Docker Layer (Humble): Runs a ROS 2 node that listens to the UDP socket, publishes a /cmd_vel topic, and monitors the /cmd_vel (teleop) topic to override it if an obstacle is detected.

Phase 2. Host Side: Data Collection (Debian)

- Since the Pi 5 has changed GPIO handling, the lgpio library is the most stable choice.

sudo apt update && sudo apt install -y python3-pip libgpiod-dev pip3 install gpiozero rpi-lgpio

- Run the Python script with the given file -

SmartCam/HiWonder_MentorPi_M1/host/socket_ultra_data.py

Phase 3. Docker Side: ROS 2 Safety Controller

- This node runs inside your MentorPi container. It acts as a "Safety Filter" between the teleop input and the motor controller. To run the teleoperation with AEB feature -

- Copy directory - SmartCam/HiWonder_MentorPi_M1/docker_MentorPi/ros2_ws to the docker container.

- source /opt/ros/humble/setup.zsh

- Go to the ros2 directory

- colcon build –symlink-install

- source install/setup.zsh

- Open two terminals in the same directory. Run

- ros2 launch controller controller.launch.py

- ros2 run teleop_ctrl tele_ultra

- Control the robot using the keyboard, following the instructions in the teleop_ctrl terminal. If there is an object close to 20 cm, the car will automatically stop